# L1.2 AV Sensors, V2X, HD Maps Quiz

1. Which perception task detects whether an object exists in the environment?
A) Object Detection
B) Classification
C) Tracking
D) Segmentation

ANS:

2. Which perception task identifies an object as a traffic sign, traffic light, or pedestrian?
A) Detection
B) Classification
C) Tracking
D) Localization

ANS:

3. Which perception task follows a moving object across time?
A) Detection
B) Classification
C) Object Tracking
D) Semantic mapping

ANS:

4. Which statement correctly describes semantic segmentation?
A) It assigns one label to an entire image.
B) It estimates only the distance to each object.
C) It assigns every pixel to a specific object instance such as car 1 or car 2.
D) It assigns each pixel to a semantic category such as road, car, or sky.

ANS:

5. How does lidar measure distance?
A) It illuminates a target with pulsed laser light and measures the reflected pulses.
B) It detects only naturally reflected visible light.
C) It triangulates the vehicle position using satellites.
D) It measures wheel rotation with magnetic encoders.

ANS:

6. Which sensor triangulates the vehicle's position using satellites?
A) Radar
B) GPS
C) Lidar
D) Ultrasonic sensor

ANS:

7. Which sensor uses high-frequency sound waves and their reflections to calculate distance?
A) Camera
B) GPS
C) Ultrasonic sensor
D) IMU

ANS:

8. What is distinctive about Tesla's current sensor configuration?
A) It uses lidar but no cameras.
B) It uses radar as its only perception sensor.
C) It uses only ultrasonic sensors for close-range perception.
D) It relies on cameras rather than lidar, radar, or ultrasonic sensors.

ANS:

9. Which of the following is a passive sensor?
A) Camera
B) Lidar
C) Radar
D) Ultrasonic sensor

ANS:

10. What distinguishes an active sensor from a passive sensor?
A) It can operate only during daylight.
B) It sends its own signal and senses the reflection.
C) It always produces a color image.
D) It determines position only from satellites.

ANS:

11. What enables stereo vision in a camera-based system?
A) One camera and one radar
B) One rotating lidar
C) Two cameras
D) One camera and one ultrasonic sensor

ANS:

12. What does a camera's field of view (FOV) specify?
A) The number of images captured per second
B) The difference between the darkest and brightest pixels
C) The maximum object-detection distance
D) The extent of the observable world visible at a given moment

ANS:

13. What happens when camera FOV is widened while resolution remains the same?
A) Image distortion becomes larger.
B) The camera becomes independent of illumination.
C) Dynamic range automatically exceeds 200 dB.
D) The image always contains more pixels.

ANS:

14. What is camera dynamic range?
A) The maximum distance at which the camera detects an object
B) The maximum difference between the darkest and lightest pixel intensities
C) The angular distance covered by two cameras
D) The number of frames stored in memory

ANS:

16. What does a lidar's point cloud describe?
A) The color distribution of the road surface
B) The vehicle's satellite coordinates
C) The audio spectrum around the vehicle
D) The 3D geometry of the surrounding environment

ANS:

17. Which is an advantage of lidar stated in the presentation?
A) It provides precise distance measurements and is independent of lighting conditions.
B) It is always the cheapest AV sensor.
C) It has higher image classification accuracy than a camera.
D) It is unaffected by all forms of bad weather.

ANS:

18. Which set contains key lidar parameters?
A) Color depth, focal length, and aperture
B) Laser beam count, rotation speed, and range distance
C) Satellite count, update rate, and map scale
D) Sound frequency, tire pressure, and wheel speed

ANS:

19. Which information can mmWave radar provide directly?
A) Only object color
B) Only semantic class labels
C) Position and relative speed
D) Lane-marking text

ANS:

20. How is long-range automotive radar characterized in the presentation?
A) Short range with a wide FOV
B) Up to 200 m with a panoramic FOV
C) A few meters with a narrow FOV
D) Up to 250 m with a narrow FOV

ANS:

21. What is a common AV application of ultrasonic sensors?
A) Parking assistance 
B) Long-range highway mapping at 250 m
C) Traffic-sign classification from color images
D) Global satellite localization

ANS:

22. Why is sensor fusion crucial for robust AV perception?
A) Every sensor has identical strengths and weaknesses.
B) Different sensors have complementary strengths and weaknesses.
C) It eliminates the need for real-time processing.
D) It guarantees that maps never become outdated.

ANS:

23. Which pair gives examples of Global Navigation Satellite Systems (GNSS)?
A) Radar and lidar
B) DSRC and C-V2X
C) GPS and BeiDou
D) IMU and RTK

ANS:

24. What position accuracy does conventional GPS typically provide?
A) Submillimeter accuracy
B) Centimeter-level accuracy under all conditions
C) Hundreds-of-meters accuracy
D) A few meters of accuracy

ANS:

26. How does RTK GPS improve the rover's position estimate?
A) By increasing camera resolution
B) By calculating and transmitting differential correction data via radio
C) By replacing satellites with ultrasonic sensors
D) By integrating only gyroscope measurements

ANS:

27. Which condition can degrade GNSS/GPS performance in an urban environment?
A) A wide camera FOV
B) High IMU update frequency
C) Signal blockage and multipath interference from tall buildings
D) Using a high-definition map

ANS:

29. What do accelerometers in an IMU measure?
A) Linear acceleration along the x, y, and z axes
B) The Earth's magnetic field only
C) Distance from laser pulse reflections
D) Radio-signal latency to a base station

ANS:

30. What do gyroscopes in an IMU measure for vehicle motion estimation?
A) Pixel intensity
B) Rotational motion around the axes
C) Satellite ephemeris data
D) Road-lane topology

ANS:

31. What additional information can a magnetometer provide when included in an IMU?
A) Object class labels
B) Long-range depth
C) Orientation information from the Earth's magnetic field
D) Traffic-light timing from infrastructure

ANS:

32. What is a major limitation of IMU-based estimation?
A) It cannot operate in poor lighting.
B) It produces data only at 10 Hz.
C) It requires a lidar point cloud to measure acceleration.
D) It is prone to drift and accumulated errors over time.

ANS:

33. Which V2X type can support a collision-avoidance system through communication between vehicles?
A) Vehicle-to-Vehicle (V2V)
B) Vehicle-to-Pedestrian (V2P)
C) Vehicle-to-Infrastructure (V2I)
D) Vehicle-to-Network (V2N)

ANS:

34. Which V2X type can send safety alerts to pedestrians and bicyclists?
A) V2V
B) V2P
C) V2I
D) V2N

ANS:

35. Adaptive traffic-light control is an example of which V2X type?
A) V2V
B) V2P
C) V2I
D) V2N

ANS:

36. Real-time traffic routing and cloud services are examples of which V2X type?
A) V2V
B) V2P
C) V2I
D) V2N

ANS:

37. Which technology is based on IEEE 802.11p at 5.9 GHz?
A) DSRC
B) RTK GPS
C) IMU
D) HD mapping

ANS:

38. For which applications was DSRC presented as particularly suitable?
A) Offline map acquisition
B) Latency-sensitive applications
C) Only entertainment downloads
D) Satellite navigation correction

ANS:

39. Which is a traditional latency-tolerant C-V2X application?
A) Immediate emergency braking
B) A do-not-pass warning
C) Over-the-Air (OTA) updates
D) Ultrasonic parking assistance

ANS:

40. Which two transmission modes are defined for C-V2X?
A) V2V and V2P
B) Camera mode and lidar mode
C) DSRC mode and GPS mode
D) V2X-Cellular and V2X-Direct

ANS:

41. How does V2X-Cellular communicate?
A) Through a cellular base station such as an eNodeB
B) Only through direct laser pulses
C) Without any network infrastructure
D) Only through traffic-sign cameras

ANS:

42. How does V2X-Direct communicate?
A) Through a cloud server and base station
B) Directly from device to device without passing through a base station
C) Only through a GNSS satellite
D) By exchanging HD-map files once per year

ANS:

43. Why can C-V2X improve warning time for a disabled vehicle beyond a blind curve?
A) It replaces all onboard perception sensors.
B) It guarantees line-of-sight communication.
C) Its longer range can deliver the warning message earlier.
D) It reduces the vehicle's camera resolution.

ANS:

44. Which consortium defines 5G V2X communications?
A) 3DPP
B) IEEE-IMU
C) HDMA
D) 5GAA

ANS:

45. Which HD-map layer contains traffic and incident information?
A) Dynamic layer
B) Localization-objects layer
C) Lane layer
D) Road layer

ANS:

46. Which items are examples of localization objects in an HD map?
A) Traffic congestion and incidents
B) Traffic signs and poles
C) Only moving pedestrians
D) Throttle and brake commands

ANS:

48. How do HD maps differ from ordinary navigation maps such as Google Maps?
A) They contain no road information.
B) They are intended only for human viewing.
C) They are updated continuously by every vehicle.
D) They provide a highly accurate, centimeter-level 3D representation designed for computer processing.

ANS:

49. How can an HD map reduce the onboard perception workload?
A) It can reduce the region of interest for detecting traffic elements.
B) It eliminates all moving objects from the environment.
C) It increases the camera's field of view.
D) It replaces vehicle control with cloud computing.

ANS:

50. Which is a disadvantage of HD maps identified in the presentation?
A) They cannot include lane geometry.
B) High acquisition cost and infrequent updates can leave them outdated.
C) They prevent localization in adverse weather.
D) They are used only by lower automation levels.

ANS: